Activities Performed For PEPS ECU,BCM ECU,ESCL ECU-

PEPS ECU (Passive Entry Passive Start)

- Developed and integrated Door Authentication and Vehicle Authentication over BLE (RN4870 module).

- Engineered the complete PEPS State Machine covering OFF, ACC, IGN RUN, and CRANK states.

- Implemented support for multiple door switches, including Driver Door, Co-Driver Door switches.

- Designed the PEPS operational flow in both forward and reverse paths to ensure full coverage of system transitions.

- Handled various failure scenarios in state transitions and authentication procedures.

- Defined and structured the PEPS CAN frame format to support all functional requirements.

- Integrated Brake Press Detection functionality using GPIO and interrupt-based mechanisms.

- Enabled Node-to-Node communication over CAN protocol between PEPS ECU, BCM ECU, and ESCL ECU.

- Monitored Engine RPM on the PEPS side to trigger appropriate alerts and state transitions.

- Implemented AES-128 encryption and decryption for secure transmission of door and vehicle passkeys.

- Developed CANTP protocol stack for UDS diagnostics, including flash read/write of DIDs and DTCs.

- Integrated the following UDS DIDs:

  - 0xF190 – VIN (Vehicle Identification Number)

  - 0xF180 – Bootloader Software ID

  - 0xF18C – ECU Serial Number

  - 0xF181 – Application Software ID

  - 0x443C – Total Number of Paired Passkeys

  - 0x443A – Door Authentication Key

  - 0x443B – Vehicle Authentication Key

  - 0x421D – BLE Module Data

  - 0x421E – Encrypted Communication Data

  - 0xF182 – Flash Read/Write Support for DIDs.

- Integrated the following UDS DTCs:

  - ERR_IN_CMD_RESPONSE=0x932011

  - DOOR_AUTH_FAIL=0x932013

  - VEHICLE_AUTH_FAIL=0x932014

- Implemented UDS services, including positive responses and Negative Response Codes (NRC).

- Integrated PIT (Periodic Interrupt Timer) for time-based operations and system timing management.

- Designed and implemented robust flash memory read/write operations.

- Integrated an OLED display to visualize real-time vehicle states, authentication results, and the CS TECH AI logo.

- Developed CAPL scripts for simulation and testing of PEPS, BCM, and Engine ECUs using Vector CANoe.

- Created and managed a DBC file representing all signals and messages for the PEPS network.

- Simulated BCM ECU behaviour on CANoe using panel controls and the DBC file.

- Implementation of Ble notification by Writing DA\r\n and CA\r\n on door characteristics and vehicle characteristics for automatic passkey

For BCM ECU-

- Developed CAN communication stack for message transmission and reception with the PEPS ECU.

- Integrated brake input detection using external interrupts for triggering state transitions.

- Implemented logic to read door ajar switch status using GPIO inputs.

- Controlled door solenoid activation based on door ajar inputs for open/close operation.

- Simulated Engine ECU functionality within the BCM by transmitting Engine RPM data.

- Toggled LED indicators (OFF, ACC, IGN RUN, CRANK) based on state information received via CAN.

For ESCL ECU-

- Implemented reception of CAN messages from the PEPS ECU to coordinate steering lock behaviour.

- Controlled the locking and unlocking of ESCL solenoids based on received command messages over CAN.

Problems faced during integrating PEPS on Lab Car-

- Ground Separation Issues in Solenoid Relay Circuit
Encountered problems due to the ground isolation between the input and output sides of the solenoid relay circuit. The input side operated on a 5V SMPS, while the solenoid (output side) was powered using a 12V SMPS, resulting in inconsistent relay behavior and triggering issues.

- Back EMF from Solenoids
Solenoids generated back electromotive force (EMF) upon deactivation, which posed a risk to the driving circuitry. This was mitigated by adding flyback diodes across the solenoids to safely dissipate the induced voltage spikes.

- Incorrect Pull Configuration on Switches.
Initially faced incorrect readings from door and vehicle switches due to improper GPIO configuration. The switches were connected to VDD, requiring the use of pull-down configuration, but were mistakenly configured with pull-up, causing logic level mismatches.